# Korg© MS2000 Program Randomizer

Copyright ©2002 John B. Davis

ms2000@pamandben.org

This application and supporting files may be freely distributed and used for non-commercial purposes.  It is provided "as-is" and the author provides no warranty expressed or implied.  Usage of this application is at the user's own risk.  The author is under no obligation to support this application in any way.

# Introduction

The Korg© MS2000 Program Randomizer application was designed to assist in creating new programs for the Korg© MS2000 Synthesizer (www.korg.com) by applying randomly chosen values for the program parameters.  It is based on the "MS2K Patch Randomizer" created by Darren Richards (darrenrichards@hotmail.com), which can be obtained from the MS2000 Yahoo Discussion Group.

The primary difference between this application and the "MS2K Patch Randomizer" is that this application allows the user to "fine-tune" which parameters are "randomized" by using standard INI files.  Each INI file contains a subset of parameter values that will be randomized and the allowable values that may be used when modifying the program.  The application also features a simulated keyboard that allows the user to quickly audition the newly generated patch by clicking the "keys" with the mouse.

It has been successfully tested under Microsoft Windows 98SE and Windows XP

Professional operating systems with generic AC97 sound cards and AMD processors.

# Usage

Upon starting the application the following window is displayed:

In order to randomize a program on an MS2000 connected to your computer you will need to first select the “MIDI output device” and “Channel”.  The “MIDI output device” might be labeled as a “Roland MPU 401” or something similar—this would be the device to which your MS2000 is connected to the computer.  The channel is simply the MIDI channel that the MS2000 is using to receive MIDI messages.

Clicking the “Randomize” button will cause the application to generate a series of MIDI controller messages with random values to the MS2000 based upon the information contained in the INI file specified by the “Current INI File” box.  The INI file can be selected by clicking the “Open File…” button, which will display a standard file selection dialog to allow the user to pick a file.

The “Show File…” button simply displays a dialog listing the application’s interpretation of the currently selected INI file by listing each parameter and the possible values for the parameter.  Eventually this feature may become an INI editing tool.

The “About…” button displays a dialog with some general application information.

The keyboard graphic along the bottom of the window will send MIDI notes to the selected MIDI device and channel using the current “Note Velocity” when clicked with the mouse.  When a key is clicked it will hold the note until the mouse button is released.

# INI File Specification

The  INI files  used by the application follow a simple convention (with italics indicating user-specified variables):

[Parameter Name]

Controller=Controller Number

Type=List or Range

This information is followed by the possible values for the parameter and is different depending on the parameter type.

The “Parameter Name” section header can be anything the user chooses, so long as the name is unique within the INI file’s section names.  It is prudent to be as descriptive as possible and use names that correspond to the actual names of the parameters they represent.

The “Controller” field specifies the MIDI controller event number that represents the program parameter.  For example, a controller of “7” will adjust the Amp Level of the program.

The “Type” field indicates if the parameter uses a range of consecutive values or if the values are specified using a list.  Some parameters, such as the Amp Level mentioned above, can have a value of anything between 0 and 127.  This type of parameter would be a “Range” type.  Other parameters have a more restricted and less homogenous set of values.  These would be a List type of parameter, where each value is literally specified in the parameter section of the INI file.  For example, the “Filter Type” parameter can be one of the following values: 0, 43, 85 or 127.  In this case the “Type” field would be “List” and each of the values would be specified as Value1=0, Value2=42, Value3=85, and so on.

The following two INI entries represent valid parameter definitions for an INI file:

[Oscillator 1 Wave]

Controller=77

Type=List

Value1=0

Value2=18

Value3=36

Value4=54

Value5=72

Value6=90

Value7=108

Value8=126

[Oscillator 1 Control 1]

Controller=14

Type=Range

RangeMin=0

RangeMax=127

Any controller value that is indicated in the INI file using the proper format will be transmitted to the synthesizer.  These are generic parameter definitions, so the application does not attempt to validate the values.  This means that you can limit or expand the INI file parameters to your liking.  If you don’t want to change the portamento, don’t add a section for it in the INI file.  If you want to have only a little portamento, add it to the INI file, but give it a very limited range.  For example, the above parameter could be modified as follows:

[Oscillator 1 Control 1]

Controller=14

Type=Range

RangeMin=9

RangeMax=30

To produce a more restricted range of values.

# Conclusion

I’ve been able to make some pretty bizarre sounds with this thing, but my limited understanding of the MS2000 keeps me from generating much that I find useful.  I hope that if you are able to create some INI files that reign-in the program parameters to a more practical level that you will share these files with the MS2000 group on the Yahoo Discussion Boards.

-------------------------------